UniFlash version 1.28 documentation

-----------------------------------

(C) 2002 Rainbow Software (Ondrej Zary)

Original version by Pascal Van Leeuwen and Galkowski Adam

"Your program just saved the day for me!

I tried to use AWDFLASH.exe to flash my new Shuttle HOT-603 v2.0(AMD640)

with the new 603WWQ07.bin BIOS and it did! The only problem was that the

BIN file wasn't complete.

Evidently this destroyed the boot block too. I tried to restore the old

BIOS to that EEPROM on a HOT-565(430TX) with all the other flash

programs I could get my hands on to no avail. I was able to flash the

HOT-603's EEPROM to 565WIQ0E.bin and it worked just fine on the HOT-565

so I knew the chip it's self was still good. I finally gave up and

ordered a replacement chip this morning. This evening while surfing I

saw a  reference to you'r program on http://ping.be and downloaded it.

With the 'Expert' mode I was able to properly flash the Winbond

W29EE011-15 chip on the HOT-565 for the HOT-603."

- Jack Freeman

0.0            DISCLAIMER

-------------------------

THIS PROGRAM COMMUNICATES WITH THE CHIPSET AND FLASH ROM CHIP AT THE LOWEST

HARDWARE LEVEL. USING THIS PROGRAM MAY BE DANGEROUS TO YOUR COMPUTER'S BIOS.

THIS PROGRAM MAY CONTAIN BUGS AND MOST OF THE SUPPORTED HARDWARE IS UNTESTED.

YOU ARE USING IT AT YOUR OWN RISK:

NOBODY EXCEPT YOU IS RESPONSIBLE FOR ANY DAMAGE THIS PROGRAM MAY CAUSE !!!

0.1            COPYRIGHT and CO.

--------------------------------

This program is public domain. You may do whatever you want with it: copy it,

delete it, rip it, modify it, etc...

If you do something with it, it would be great if you credited us.

1.0            UniFlash? UNIFLASH??? What the HELL is it ??!!!

--------------------------------------------------------------

UniFlash is a flash program, used to write a new BIOS on your FLASH ROM Chip.

What's new there? , you may ask, there are already programs like this on

Internet made by BIG BRANDS like AMI, AWARD or MR. Well... there is only a

little difference: COMPLETE, WORKING SOURCE CODE IN PASCAL IS GIVEN FOR FREE!!

UniFlash is made to be universal - this can be useful when you're hot-flashing

your bios in another motherboard.

Another thing: this flasher is mainly destinated for fearless people and

experienced users. If it's your first adventure with flashing your BIOS, we

recommend you strongly using these brand flashers. You'll have to understand

we are just experienced computer users/programmers and we haven't materially

the possibility to test every routine and chips we haven't. That's the main

reason we release it. We hope people may help us in debugging and testing

routines.

NOTE by PvL: Brand name flashers have a tendency to be unstable (this is

especially true for AWDFLASH, which f*cked up my BIOS twice

so far, leaving me no other choice than to hot-flash) so if

you happen to be the lucky owner of one of the TESTED flash

chips I can really recommend using UNIFLASH instead ...

NOTE by Rainbow:

AWDFLASH IS REALLY BAD. When I was flashing BIOS on Abit SA6R, the verify

failed. I retried flashing about 20 times, then it succeeded. But motherboard

died after reboot. Luckily, after powering down and back up, BootBlock BIOS

worked and I was able to flash the BIOS correctly from there.

2.0            What hardware do I need for UniFlash ?

-----------------------------------------------------

Here's the minimum configuration you need to run UniFlash:

-At least a 386 (The whole thing works in flat real mode)

-RAM at least 2 times bigger than Flash ROM size, but less than 2GB

-MS DOS 4.0 and higher

-Known Flash ROM chip (see section 3.0)

-Supported chipset (see below) - note that all of them are PCI based

,----------------------------------------------------------------------------,

| Don't forget to disable "Flash write protect" - either in system BIOS or   |

| using a jumper on the motherboard.                                         |

`----------------------------------------------------------------------------'

Here's a list of chipsets suported by Uniflash:

* means tested and functional,

# means tested, but doesn't work the way it should

unmarked chipsets are not tested

,----------------------------------------------------------------------------,

| Mfg     | Chipset(s)           | Comments                                  |

`---------|----------------------|-------------------------------------------'

Intel    |*430FX                | No comments It works in 99% of cases

|*430HX                | 450 & 434 not tested

|*430VX                | 440GX and 440FX untested

|*430TX                |

|*430MX                |

| 440FX                |

|*440LX/EX             |

| 450KX/GX             |

| 434LX/NX             |

|*440BX/ZX             |

| 440GX                |

| i810                 | Hub architecture chipsets (i8xx) are

| i810E                | not tested.

| i810E2               |

| i815                 |

| i815E(P)             |

| i815EM               |

| i820                 |

| i820E                |

| i830MP               |

| i840                 |

| i850                 |

| i860                 |

| E7500                |

----------|----------------------|--------------------------------------------

VIA      | Appollo Master       |

| Apollo VP            |

|*Apollo VPX           |

|*Apollo VP2           |

| Apollo VP3           |

|*Apollo MVP3          |

| Apollo P6            |

| Apollo MVP4          | VIA chipsets are weird - some work, some

| Apollo Pro           | don't.

| Apollo Pro Plus/133  |

|*Apollo Pro 133A      | If you have "Flash Write protect" in BIOS,

| Apollo ProMedia      | disable it and it should work.

| Apollo Pro 133Z/PM133|

| Apollo KX133         |

|*Apollo KT133(A)      |

| ProSavage KM133(A)   |

|*Apollo KT266(A)      |

| Apollo Pro 266       | These North Bridges are detected, but

| ProSavage P4M266     | aren't used in any VIA's chipset (??):

| P4X266(A)/P4X333     | VT8653, VT8662, VT8615, VT8361, VT3133

----------|----------------------|--------------------------------------------

AMD      |*AMD 640 (same as VIA | Works just fine

| Apollo VP2)          |

| AMD 750              | UniFlash supports true AMD chipsets since

|#AMD 760              | version 1.20. Their registers are very

| AMD 760MP            | similar to VIA. AMD 760 should work with

| AMD 760MPX           | both VIA and AMD (does it really exist?)

|                      | south bridges.

----------|----------------------|--------------------------------------------

SiS      | 85C496+497           | I got the datasheets, but not from SiS.

|#501/5101/5501        |

| 5511                 | These chipsets are really crappy - to

| 5571                 | support 12 chipsets, I needed to write

|*5591/5592            | 5 different methods of enabling Flash ROM

| 5596                 | write access. (The more code, the more

|*5597/5598/5581/5120  | bugs :-)

| 530                  | All SiS South Bridges have the same

| 540                  | device ID $0008 (SiS 540 and 630 can have

| 600                  | either $0008 or $0018 - it can be

|*620                  | configured(!!!) through registers.

|*630                  |

| 635                  |

| 640                  | Boards with the new chipsets (540 and 630

| 645                  | and up) have often ITE IT8705F LPC Super IO

| 645DX                | (also called SiS 950 sometimes) which

| 650                  | allows to use regular Flash ROM on LPC bus.

| 730                  | It's tested and works.

|*735                  |

| 740                  |

----------|----------------------|--------------------------------------------

ALi      |*FinALi 486           | FinALi has very weird Flash ROM access but

|*Aladdin Pro II       | finally it works correctly.

| Aladdin III          | All chipsets with M1533/M1543/M1535 South

| Aladdin IV/IV+       | Bridge should work.

|*Aladdin V            | Unlisted chipsets are not fully detected,

| Aladdin Pro IV       | but should work if they use M1533/43/35 or

| ALiMAGiK 1           | M1523 South Bridge.

----------|----------------------|--------------------------------------------

SMSC     | VictoryBX-66         | i82443BX North Bridge with SMSC SLC90E66

|                      | South Bridge (UDMA-66 support)

|                      | Untested.

----------|----------------------|--------------------------------------------

Winbond/ | W83C553F South Bridge| I don't know if this one was ever used.

Symphony |                      | (No known chipset with this SB)

Labs     |                      | Untested, of course.

----------|----------------------|--------------------------------------------

ITE      | IT8871F/2F and       | Don't know where used, untested.

| IT8888F South Bridges| (No known chipset with these SBs)

|*IT8700F/8705F        | LPC Super I/Os with Flash ROM support

| IT8710               | (used with SiS 540, SiS 630 and newer)

----------|----------------------|--------------------------------------------

Reliance/| ServerSet III LE     | All chipsets with OSB4 South Bridge should

Server   | ServerSet III HE     | work. Untested.

Works    |                      | I don't have any official info.

----------|----------------------|--------------------------------------------

National/| Vesuvius-LS          | Never seen this one.

PicoPower|                      | Untested

----------|----------------------|--------------------------------------------

National/| Geode GX1 (MediaGX)  | Supported with Geode CS5530(A) South Bridge

Cyrix    |                      | (Cyrix CX5530(A))

----------|----------------------|--------------------------------------------

OPTi     | Vendetta 82C750      | Not tested

----------|----------------------|--------------------------------------------

HiNT     | VXPro-II             | Used on some PC Chips boards (like M559)

|                      | I think that it's Utron UC85C501/502

|                      | Not tested

----------|----------------------|--------------------------------------------

UMC      |*UM8881F+UM8886BF     | Don't have any datasheets, tested and works

------------------------------------------------------------------------------

PC100 BXCel is relabelled ALi Aladdin Pro II on PC Chips M726 motherboard.

The version 1.16d+ includes support for AMI's Flash Interface. It allows you

to flash even if you have an unknown chipset (new AMI WinBIOS is necessary to

use this feature). Starting from version 1.24, you must force using this mode,

if you want to use it by giving the -AMI parameter. This is neccessary because

it often doesn't work and hangs, preventing users from making BIOS backup.

2.1            Tested PCI cards

-------------------------------

* means tested and functional

# means tested, but doesn't work

* Adaptec 19160 (AIC-7892B chip) - Winbond W29EE512

# AdvanSys ASB-3940UA

# ATI Rage Magnum AGP

# Matrox Millennium PCI

3.0            What flash chips are supported by UniFlash ?

-----------------------------------------------------------

UniFlash supports the following flash chips:

* means tested and functional,

# means tested, but doesn't work the way it should,

unmarked chips are not tested (note that a lot of chips are very similar to

one another though, so if one of them works then the rest should work fine

too)

,----------------------------------------------------------------------------,

| Flash ROM                       | Size                                     |

`---------------------------------|------------------------------------------'

Alliance Semiconductor          |

----------------------          |

AS29F040                        | 512KB

AS29F002T                       | 256KB

AS29F002B                       | 256KB

AS29F200T                       | 256KB

AS29F200B                       | 256KB

AS29LV400T                      | 512KB

AS29LV400B                      | 512KB

AS29LV800T                      | 1024KB

AS29LV800B                      | 1024KB

----------------------------------|-------------------------------------------

AMD                             |

---                             |

Am28F256                        | 32KB

Am28F256A                       | 32KB

Am28F512                        | 64KB

Am28F512A                       | 64KB

Am28F010                        | 128KB

Am28F010A                       | 128KB

Am28F020                        | 256KB

Am28F020A                       | 256KB

*Am29F010(A/B)                   | 128KB

Am29F040(B)                     | 512KB

Am29F080(B)                     | 1024KB

Am29F016(B)                     | 2048KB

Am29F017B                       | 2048KB

Am29F032B                       | 4096KB

Am29F100T                       | 128KB

Am29F100B                       | 128KB

Am29F200AT/BT                   | 256KB

Am29F200AB/BB                   | 256KB

Am29F400AT/BT                   | 512KB

Am29F400AB/BB                   | 512KB

Am29F800(B)T                    | 1024KB

Am29F800(B)B                    | 1024KB

Am29F160DT                      | 2048KB

Am29F160DB                      | 2048KB

*Am29F002(N)(B)T                 | 256KB

Am29F002(N)(B)B                 | 256KB

Am29F004BT                      | 512KB

Am29F004BB                      | 512KB

Am29LV010B                      | 128KB

Am29LV040B                      | 512KB

Am29LV017B                      | 2048KB

Am29LV033C                      | 4096KB

Am29LV065D                      | 8192KB

Am29LV081B                      | 1024KB

Am29LV001BT                     | 128KB

Am29LV001BB                     | 128KB

Am29LV002(B)T                   | 256KB

Am29LV002(B)B                   | 256KB

Am29LV004(B)T                   | 512KB

Am29LV004(B)B                   | 512KB

Am29LV008(B)T                   | 1024KB

Am29LV008(B)B                   | 1024KB

Am29LV200(B)T                   | 256KB

Am29LV200(B)B                   | 256KB

Am29LV400(B)T                   | 512KB

Am29LV400(B)B                   | 512KB

Am29LV800BT                     | 1024KB

Am29LV800BB                     | 1024KB

Am29LV160BT/DT                  | 2048KB

Am29LV160BB/DB                  | 2048KB

Am29LV116BT                     | 2048KB

Am29LV116BB                     | 2048KB

Am29LV320DT                     | 4096KB

Am29LV320DB                     | 4096KB

Am29SL800CT                     | 1024KB

Am29SL800CB                     | 1024KB

Am29SL160CT                     | 2048KB

Am29SL160CB                     | 2048KB

Am29PL160CB                     | 2048KB

----------------------------------|-------------------------------------------

AMIC                            |

----                            |

A29010                          | 128KB

A29040                          | 512KB

A29001(1)T                      | 128KB

A29001(1)B                      | 128KB

*A29002(1)T                      | 256KB

A29002(1)B                      | 256KB

A29400T                         | 512KB

A29400B                         | 512KB

----------------------------------|-------------------------------------------

ASD                             |

---                             |

*AE29F1008 = Winbond W29EE011    | 128KB

*AE29F2008 = Winbond W29C020(C)  | 256KB

----------------------------------|-------------------------------------------

Atmel                           |

-----                           |

AT29C256 or AT29C257            | 32KB

AT29C512                        | 64KB

*AT29C010(A)                     | 128KB

AT29C020(A)                     | 256KB

AT29C040A                       | 512KB

AT29LV256                       | 32KB

AT29LV512                       | 64KB

AT29xV010(A) series             | 128KB

AT29xV020 series                | 256KB

AT29xV040A series               | 512KB

AT49x512 series                 | 64KB

AT49x001(N)T series             | 128KB

AT49x001(N) series              | 128KB

*AT49x002(N)T series             | 256KB

AT49x002(N) series              | 256KB

AT49x008AT series               | 1024KB

AT49x008(A) series              | 1024KB

AT49x010 series                 | 128KB

AT49x020 series                 | 256KB

AT49x040 series                 | 512KB

AT49x080 series                 | 1024KB

AT49x080T series                | 1024KB

AT49x4096(A) series             | 512KB

AT49x8011T series               | 1024KB

AT49x8011 series                | 1024KB

AT49x160/161(4)T series         | 2048KB

AT49x160/161(4) series          | 2048KB

AT49BV320T/321T                 | 4096KB

AT49BV320/321                   | 4096KB

----------------------------------|-------------------------------------------

Bright Microelectronics         |

-----------------------         |

BM29F040  = Hyundai HY29F040    | 512KB

BM29F400T = Hyundai HY29F400T   | 512KB

BM29F400B = Hyundai HY29F400B   | 512KB

----------------------------------|-------------------------------------------

Catalyst                        |

--------                        |

CAT28F512                       | 64KB

*CAT28F010                       | 128KB

CAT28F020                       | 256KB

*CAT28F001T                      | 128KB

*CAT28F001B                      | 128KB

CAT28F002T                      | 256KB

CAT28F002B                      | 256KB

CAT28F150T                      | 192KB

CAT28F150B                      | 192KB

CAT29F002T                      | 256KB

CAT29F002B                      | 256KB

----------------------------------|-------------------------------------------

EON                             |

---                             |

EN29F040                        | 512KB

EN29F080                        | 1024KB

*EN29F002T                       | 256KB

EN29F002B                       | 256KB

EN29F800T                       | 1024KB

EN29F800B                       | 1024KB

----------------------------------|-------------------------------------------

Fujitsu                         |

-------                         |

MBM29F002TC                     | 256KB

MBM29F002BC                     | 256KB

MBM29F200TC                     | 256KB

MBM29F200BC                     | 256KB

MBM29F400TC                     | 512KB

MBM29F400BC                     | 512KB

MBM29F040C                      | 512KB

MBM29F033C                      | 4096KB

MBM29LV160TE                    | 2048KB

MBM29LV160BE                    | 2048KB

MBM29PL160TD                    | 2048KB

MBM29PL160BD                    | 2048KB

----------------------------------|-------------------------------------------

H.T.                            |

----                            |

*M21561 = Macronix MX28F1000P    | 128KB

*M21032T = Macronix MX28F1000P   | 128KB

*M20271H = Macronix MX28F1000P   | 128KB

----------------------------------|-------------------------------------------

Hyundai                         |

-------                         |

HY29F040                        | 512KB

HY29F040A                       | 512KB

HY29F080                        | 1024KB

HY29F002T                       | 256KB

HY29F002B                       | 256KB

HY29F400T                       | 512KB

HY29F400B                       | 512KB

HY29F800T                       | 1024KB

HY29F800B                       | 1024KB

HY29LV160T                      | 2048KB

HY29LV160B                      | 2048KB

----------------------------------|-------------------------------------------

IMT                             |

---                             |

IM29F001T                       | 128KB

IM29F001B                       | 128KB

IM29F002T                       | 256KB

IM29F002B                       | 256KB

IM29F004T                       | 512KB

IM29F004B                       | 512KB

IM29LV004T                      | 512KB

IM29LV004B                      | 512KB

----------------------------------|-------------------------------------------

Intel                           |

-----                           |

28F256(A)                       | 32KB

28F512                          | 64KB

*28F010                          | 128KB

28F020                          | 256KB

*28F001BX/BN-T                   | 128KB

28F001BX/BN-B                   | 128KB

28F002-T series                 | 256KB

28F002-B series                 | 256KB

#28F004-T series                 | 512KB

28F004-B series                 | 512KB

28F008-T series                 | 1024KB

28F008-B series                 | 1024KB

28F008SA                        | 1024KB

28F004Sx series                 | 512KB

28F008Sx series                 | 1024KB

28F016Sx series                 | 2048KB

28F016S5                        | 2048KB

28F004B3-T                      | 512KB

28F004B3-B                      | 512KB

28F008B3-T                      | 1024KB

28F008B3-B                      | 1024KB

28F016B3-T                      | 2048KB

28F016B3-B                      | 2048KB

28F320J5                        | 4096KB

28F320J3A                       | 4096KB

28F640J5                        | 8192KB

28F640J3A                       | 8192KB

28F128J3A                       | 16384KB

82802AB (Firmware Hub)          | 512KB

82802AC (Firmware Hub)          | 1024KB

----------------------------------|-------------------------------------------

ISSI                            |

----                            |

IS28F010                        | 128KB

IS28F020                        | 256KB

----------------------------------|-------------------------------------------

Macronix                        |

--------                        |

MX28F1000                       | 128KB

*MX28F1000P                      | 128KB

MX28F2000P                      | 256KB

MX28F2000T                      | 256KB

MX28F002T                       | 256KB

MX28F002B                       | 256KB

MX29F040                        | 512KB

MX29F016                        | 2048KB

MX29F001T                       | 128KB

*MX29F001B                       | 128KB

*MX29F002(N)T                    | 256KB

MX29F002(N)B                    | 256KB

MX29F022T                       | 256KB

MX29F022B                       | 256KB

MX29F004T                       | 512KB

MX29F004B                       | 512KB

MX29F100T                       | 128KB

MX29F100B                       | 128KB

MX29F200T                       | 256KB

MX29F200B                       | 256KB

MX29F400T                       | 512KB

MX29F400B                       | 512KB

MX29F800T                       | 1024KB

MX29F800B                       | 1024KB

MX29F1610A                      | 2048KB

MX29F1615                       | 2048KB

MX29LV040                       | 512KB

MX29LV081                       | 128KB

MX29LV004T                      | 512KB

MX29LV004B                      | 512KB

MX29LV008T                      | 1024KB

MX29LV400T                      | 512KB

MX29LV400B                      | 512KB

MX29LV800T                      | 1024KB

MX29LV800B                      | 1024KB

MX29LV160T                      | 2048KB

MX29LV160B                      | 2048KB

MX29L1611                       | 2048KB

MX29L1611G                      | 2048KB

MX29L3211                       | 4096KB

----------------------------------|-------------------------------------------

Megawin                         |

-------                         |

MM29F040 = AMD Am29F040(B)/5V   | 512KB

----------------------------------|-------------------------------------------

Micron                          |

------                          |

MT28F004-T = Intel 28F004-T     | 512KB

MT28F004-B = Intel 28F004-B     | 512KB

MT28F400-T = Intel 28F400-T     | 512KB

MT28F400-B = Intel 28F400-B     | 512KB

MT28F008-T = Intel 28F008-T     | 1024KB

MT28F008-B = Intel 28F008-B     | 1024KB

MT28F800-T = Intel 28F800-T     | 1024KB

MT28F800-B = Intel 28F800-B     | 1024KB

----------------------------------|-------------------------------------------

Mosel Vitelic                   |

-------------                   |

V29C51000T                      | 64KB

V29C51000B                      | 64KB

V29C51001T                      | 128KB

V29C51001B                      | 128KB

V29C51002T                      | 256KB

V29C51002B                      | 256KB

V29C51004T                      | 512KB

V29C51004B                      | 512KB

V29C51400T                      | 512KB

V29C51400B                      | 512KB

V29LC51000                      | 64KB

V29LC51001                      | 128KB

V29LC51002                      | 256KB

V29C31004T                      | 512KB

V29C31004B                      | 512KB

V29C31400T                      | 512KB

----------------------------------|-------------------------------------------

NexFlash Technologies           |

---------------------           |

*NX29F010  = AMD Am29F010        | 128KB

NX29F400B = AMD Am29F400B       | 512KB

----------------------------------|-------------------------------------------

PMC                             |

---                             |

Pm39LV512R                      | 64KB

Pm39LV010R                      | 128KB

Pm29F002T                       | 256KB

Pm29F002B                       | 256KB

Pm29F004T or Pm29LV104T         | 512KB

Pm29F004B or Pm29LV104B         | 512KB

----------------------------------|-------------------------------------------

SST                             |

---                             |

28EE010/011                     | 128KB

29EE512                         | 64KB

*29EE010                         | 128KB

*29EE020                         | 256KB

28x040(A) series                | 512KB

29x512 series                   | 64KB

29x010 series                   | 128KB

29x020 series                   | 256KB

29SF512                         | 64KB

29SF010                         | 128KB

29SF020                         | 256KB

29SF040                         | 512KB

29VF512                         | 64KB

29VF010                         | 128KB

29VF020                         | 256KB

29VF040                         | 512KB

*49LF020                         | 256KB

49LF030                         | 384KB

49LF040                         | 512KB

49LF080                         | 1024KB

49LF002(A) (Firmware Hub)       | 256KB

49LF003A (Firmware Hub)         | 384KB

49LF004 (Firmware Hub)          | 512KB

49LF004A (Firmware Hub)         | 512KB

49LF008 (Firmware Hub)          | 1024KB

49LF008A (Firmware Hub)         | 1024KB

39SF512                         | 64KB

39SF010(A)                      | 128KB

*39SF020(A)                      | 256KB

39SF040                         | 512KB

39xF512 series                  | 64KB

39xF010 series                  | 128KB

*39xF020 series                  | 256KB

39xF040 series                  | 512KB

39xF080 series                  | 1024KB

39xF016 series                  | 2048KB

----------------------------------|-------------------------------------------

STMicroelectronics              |

------------------              |

M28F256                         | 32KB

M28F512                         | 64KB

M28F101                         | 128KB

M28F102                         | 256KB

M28F201                         | 256KB

M29F512B                        | 64KB

M29F010B                        | 128KB

M29F040(B)                      | 512KB

M29F080A                        | 1024KB

M29F016B                        | 2048KB

M29F002(B)(N)T                  | 256KB

M29F002(B)B                     | 256KB

M29F100(B)T                     | 128KB

M29F100(B)B                     | 128KB

M29F200(B)T                     | 256KB

M29F200(B)B                     | 256KB

M29F400(B)T                     | 512KB

M29F400(B)B                     | 512KB

M29F800AT                       | 1024KB

M29F800AB                       | 1024KB

M29F160BT                       | 2048KB

M29F160BB                       | 2048KB

M29W512B                        | 64KB

M29W010B                        | 128KB

M29W022BT                       | 256KB

M29W022BB                       | 256KB

M29W040(B)                      | 512KB

M29W200BT                       | 256KB

M29W200BB                       | 256KB

M29W400(B)T                     | 512KB

M29W400(B)B                     | 512KB

M29W800(A)T                     | 1024KB

M29W800(A)B                     | 1024KB

M29W160BB/DB                    | 2048KB

M29W004(B)T                     | 512KB

M29W004(B)B                     | 512KB

M29W008(A)T                     | 1024KB

M29W008(A)B                     | 1024KB

M50FW040 (FWH)                  | 512KB

M50FW080 (FWH)                  | 1024KB

M50LPW080 (LPC)                 | 1024KB

----------------------------------|-------------------------------------------

SyncMOS                         |

-------                         |

F29C51001T = Mosel V29C51001T   | 128KB

F29C51001B = Mosel V29C51001B   | 128KB

#F29C51002T = Mosel V29C51002T   | 256KB

F29C51002B = Mosel V29C51002B   | 256KB

F29C51004T = Mosel V29C51004T   | 512KB

F29C51004B = Mosel V29C51004B   | 512KB

F29C31004T = Mosel V29C31004T   | 512KB

F29C31004B = Mosel V29C31004B   | 512KB

F29C31400T = Mosel V29C31400T   | 512KB

----------------------------------|-------------------------------------------

Texas Instruments               |

-----------------               |

TMS29xF040                      | 512KB

TMS28F002A-T = Intel 28F002-T   | 256KB

TMS28F002A-B = Intel 28F002-B   | 256KB

TMS28F200A-T = Intel 28F200-T   | 256KB

TMS28F200A-B = Intel 28F200-B   | 256KB

TMS28F200BZT = Intel 28F200-T   | 256KB

TMS28F200BZB = Intel 28F200-B   | 256KB

TMS28F400A-T = Intel 28F400-T   | 512KB

TMS28F400A-B = Intel 28F400-B   | 512KB

TMS28F400BZT = Intel 28F400-T   | 512KB

TMS28F400BZB = Intel 28F400-B   | 512KB

TMS28F004A-T = Intel 28F004-T   | 512KB

TMS28F004A-B = Intel 28F004-B   | 512KB

TMS28F008A-T = Intel 28F008-T   | 1024KB

TMS28F008A-B = Intel 28F008-B   | 1024KB

TMS28F800A-T = Intel 28F800-T   | 1024KB

TMS28F800A-B = Intel 28F800-B   | 1024KB

TMS28F010A/B = Intel 28F010     | 128KB

TMS28F020    = Intel 28F020     | 256KB

TMS28F512A   = Intel 28F512     | 64KB

TMS29F002RT  = AMD Am29F002T    | 256KB

TMS29F002RB  = AMD Am29F002B    | 256KB

TMS29F008T   = AMD Am29F800T    | 1024KB

TMS29F008B   = AMD Am29F800B    | 1024KB

TMS29F010    = AMD Am29F010     | 128KB

TMS29F040    = AMD Am29F040     | 512KB

TMS29F400T   = AMD Am29F400AT   | 512KB

TMS29F400B   = AMD Am29F400AB   | 512KB

TMS29F800T   = AMD Am29F800T    | 1024KB

TMS29F800B   = AMD Am29F800B    | 1024KB

TMS29LF008T  = AMD Am29LV008T   | 1024KB

TMS29LF008B  = AMD Am29LV008B   | 1024KB

TMS29LF800T  = AMD Am29LV800BT  | 1024KB

TMS29LF800B  = AMD Am29LV800BB  | 1024KB

----------------------------------|-------------------------------------------

Winbond                         |

-------                         |

*W29x010/011 series              | 128KB

*W29C020(C)                      | 256KB

W29C040                         | 512KB

W29EE512                        | 64KB

*W49F002U                        | 256KB

W49V002(A)                      | 256KB

W49V002F(A) (Firmware Hub)      | 256KB

------------------------------------------------------------------------------

3.1            Flash ROM pinout and notes

-----------------------------------------

Some 12V Flash ROMs require 12V voltage not only to flash, but also to detect.

If you're trying to flash 12V chip, first check that you have 12V on the VPP

pin in the socket on motherboard. Flash write protect jumper usually removes

12V from this pin.

When some parts of the chip are write protected, 12V on VPP/RESET pin will

usually allow these protected parts to be flashed (unless the chip has N in

the name - e.g. MX29F002NT - this means that RESET pin is not connected).

Be sure that Flash ROM is inserted the correct way in the socket (key on the

chip must match with key on the socket) BEFORE powering up (if you're

hot-flashing, double check chip orientation before inserting it into socket).

Inserting it wrong way will DESTROY the chip - VCC will be swapped with GND

and if there's 12V on the VPP pin, it will be connected to Q3 data pin...

Flash ROM pinout (32-pin DIP):

_____KEY_____

VPP -|     \_/     |- VCC     VPP = Programming voltage (12V)

A16 -|             |- #WE     VCC = Operating voltage (5V)

A15 -|             |- A17     GND = Ground

A12 -|             |- A14     A0..A17 = Address input pins

A7 -|             |- A13     Q0..Q7  = Data input/output pins

A6 -|             |- A8      #WE = Write enable

A5 -|             |- A9      #OE = Output enable

A4 -|             |- A11     #CE = Chip enable

A3 -|             |- #OE     (# means active low signal)

A2 -|             |- A10

A1 -|             |- #CE     Notes:

A0 -|             |- Q7        - some chips have RESET instead of VPP

Q0 -|             |- Q6        - this is 256KB chip

Q1 -|             |- Q5        - 512KB chips have A18 instead of VPP

Q2 -|             |- Q4        - 128KB chips have NC instead of A17

GND -|_____________|- Q3        - 64KB chips have NC instead of A17 & A16

4.0            Help! I don't understand the sources !!!

-------------------------------------------------------

If you have problems understanding something mail me (address at the end).

Have a look also on the ADDCHIP.FAQ file. It explains how to add new chips

plus some details.

5.0            ADVANCED submenu

-------------------------------

There's ADVANCED submenu in UniFlash, which contains possibly dangerous

functions. If you're just a user don't enter it (unless you have to ...). This

menu contains functions that allow you to overwrite the bootblock (which is

not normally possible).

WARNING: NEVER overwrite the bootblock if you're not sure if support for your

Flash ROM in UniFlash is working! If you're trying to fix a bad-flashed BIOS,

ALWAYS try first to FLASH WITHOUT OVERWRITING THE BOOTBLOCK! If flashing

completes successfully (no verify errors) but BIOS doesn't work because of

bootblock incompatibility with the BIOS, THEN overwrite the bootblock.

NOTE: If you want to change between AMI <-> Award BIOS or flash BIOS for

another motherboard, ALWAYS flash including bootblock.

5.1            Making an emergency disk

---------------------------------------

It is always nice to have an emergency disk handy in case something goes

wrong. You see, most flash BIOS's incorporate something called a 'boot block'

and UNIFLASH will not normally overwrite it so it remains intact.

The boot block contains just enough code to boot up your computer with basic

peripheral support to enable you to flash in a working BIOS in case something

went wrong.

To make an emergency disk one would do the following:

-make a DOS (or Win95/Win98, NOT ANY OTHER) system disk using "format a: /S"

-copy uniflash to that disk

-copy a working bios image (preferably a backup that was made while the system

was still a-ok) to the disk

-make an AUTOEXEC.BAT containing:

@UNIFLASH -E <name of BIOS image file>

NOTE: Whatever you do, do *NOT* load HIMEM.SYS. The boot block lacks support

routines that HIMEM.SYS requires, resulting in very nice crashes.

And I know from experience ... !! In general, keep the boot disk as

clean as possible, remove drvspace.bin and dblspace.bin.

Make sure UNIFLASH and the BIOS image are on the disk as the boot block only

supports the floppy drive, you won't be able to access any harddrive !!

When you boot from this emergency disk the working BIOS image will

automatically (without any intervention by you) be flashed into the flash ROM

by UNIFLASH. This is very convenient as the boot block has no support for PCI

or AGP video cards, so you won't be able to SEE anything in the process unless

you replace your PCI videocard with an old ISA one (hint!). If you have

motherboard with i815(E) chipset and you're using onboard video, it will

probably not work in boot block mode (because graphics BIOS is stored in main

Flash ROM chip) - try to put an AGP card in (there are no ISA slots).

UNIFLASH will give you some audible feedback, though:

first short beep: UNIFLASH invoked, reading BIOS image

second short beep: done reading BIOS image, starting to flash

chime sound: done flashing, reset your system

long continuous beep: some error occured, press any key to retry operation

5.2            Command-line parameters

--------------------------------------

UNIFLASH [-H|-?|[-E fname][-PCIOR [BUS DEV FUN]|-AMI|-BASE xxxxx]

[-SAVE fname|-CHIPLIST|-FORCE xxxx][-REPAIR][-LOG][-MONO]

[-CMOSS fname][-CMOSR fname][-CMOSC]][-QUIT][-REBOOT]

-H or -?        Shows command-line parameters (this list)

-E fname        Flashes bios image from file fname to Flash ROM without any

user intervention

-PCIOR          Flash PCI option ROM instead of system ROM

[BUS DEV FUN] Manually specify PCI device (decimal numbers)

-AMI            Use AMI Flash Interface

-CHIPLIST       Show list of supported flash chips with their numbers

-FORCE xxxx     Force using routines for chip xxxx (4 hex digits)

-BASE xxxxx     Set ROM Base to address xxxxx (at least 5 hex digits)

-REPAIR         Repair erased chip ID (Winbond and SST chips)

-SAVE fname     Write current bios image to file fname

-CMOSS fname    Save current CMOS settings to file fname

-CMOSR fname    Restore CMOS settings from file fname

-CMOSC          Clears CMOS settings

-QUIT           Quits (e.g. UNIFLASH -SAVE BACKUP.BIN -QUIT)

-REBOOT         Reboot after flashing (use together with -E)

Parameters are executed in the same order as they are entered on the command

line - so you can use e.g. "uniflash -cmoss cmos.bak -cmosc" to save CMOS

settings to file called cmos.bak and then clear the CMOS.

Some parameters can't be used together - e.g. -AMI can't be used with -PCIOR.

-E: Flashes BIOS image from file fname including bootblock without

any user input

-PCIOR: Selects ROM on a PCI device instead of system ROM. Works only with

some cards. You can optionally specify the device you want to use.

(e.g. -PCIOR 0 3 1 = BUS 0, DEVICE 3, FUNCTION 1)

-AMI: Uses interface supported by some AMI BIOSes instead of detecting the

chipset and writing to chipset registers to disable write protection.

(I haven't seen this working anywhere yet...)

-CHIPLIST: Displays a list of supported chips with their IDs which are needed

for use with -FORCE parameter.

-FORCE: Disable Flash ROM autodetection and set chip type to the one specified

by following ID (4 hex digits) - see -CHIPLIST parameter.

-BASE: Set ROM base address manually and allow running without PCI bus.

Useful for memory-mapped flash programming cards with old systems.

-REPAIR: ID on some Winbond and SST chips can be changed which can cause the

chip not being recognized by any software (including BIOS - AMI BIOS

does not work in that chip, Award BIOS displays Unknown Flash Type).

This option can change the ID on these chips. Because UniFlash does

not know the correct ID, you must specify it using -FORCE parameter.

-SAVE: Save current BIOS to file fname.

-CMOSS: Save current CMOS settings to file fname. CMOS memory size is detected

automatically.

-CMOSR: Restore CMOS settings from file fname. CMOS memory size is detected

automatically.

-CMOSC: Clear CMOS memory.

6.0            How to support UniFlash?

---------------------------------------

If you want to help me in adding support for new chips, in testing the whole

thing or bugfixing, mail me, I'll put your name in credits at the end.

I want your opinions & experiences to make flashing BIOS an easy and safe

operation, where you doesn't need tons of flashers (if you multiply the

number of chipsets by the number of Flash ROM memories available, you'll

understand better what we're talking about).

7.0            Credits

---------------------------------------

Ondrej Zary (Rainbow Software) - Continues to develop this program after the

authors (below) stopped the development. Done all that is new after v1.17b.

Added tons of Flash ROMs and chipsets, fixed a lot of bugs, got datasheets

from ALi.

Pascal van Leeuwen - Made first design, chipset handling routines, a primitive

write function for SST, WINBOND and INTEL, added flat real mode, etc...

Improved SST & Winbond routines, made AMD routines work, added Atmel support,

added lots & lots of flash chips (of which most are currently untested ...)

Added reliability improvements (backup of old BIOS, verify after write)

Tested both Winbond & SST chips, became an expert at hot-flashing ;-)

Galkowski Adam - Initiated the UniFlash project, made modular structure, made

the current design, added functions handling DMI, PnP and bootblock, added

expert mode, made the previous crappy AMD support, wrote this doc, tested SST

routines, had multiple rendez-vous with "Award BootBlock BIOS",

advertised, etc...

8.0            How to contact me

--------------------------------

You can contact me (Rainbow Software) by:

web    : http://rainbow.ht.st

web    : http://rainbow.host.sk

(new versions should be at both addresses above)

e-mail : rainbow@host.sk

When sending reports that something doesn't work, always send the log file!

9.0            Revision history

-------------------------------

1.28       -fixed -SAVE paramter (it was completely broken)

-display Chipset and Flash ROM when only -QUIT parameter specified

-fixed bug causing reset of ITE LPC Super I/Os (resulting in floppy

and COM&LPT ports not working until reboot)

-fixed Macronix chip detection

-fixed Atmel AT29Cxxx detection (again, it really works now)

-fixed Atmel programming routine (chip remained unprotected)

-fixed some SiS chipset support

-fixed Utron 128KB support (again)

-fixed showing SUCCESS when bootblock mismatch and user answered NO

-fidex the detection routine erased vendor and device IDs from some

Winbond and SST chips (detected properly once but then unknown,

BIOS displayed "Unknown Flash Type" message)

-added repair feature to repair the erased IDs (-repair parameter)

-added support for ALi FinALi 486 chipset

-enable/disable menu items after redetect with/without Flash ROM

-verify now shows unchanged areas

-removed $4B and $D5 IDs for Macronix MX28F1000P (they were there

because of the detection bug which is now fixed)

-added logging for north bridge methods

-added support for UMC UM8881F+UM8886BF chipset (no datasheets,

reverse engineering used)

-fixed hang when running under Award BootBlock BIOS with

uninitialized or absent graphics card

-added user friendly filename editing, default file names when

saving a file and file browser when opening a file (press ENTER on

the prompt, TAB to change drive)

-added I/O error checking for all file operations

-added -REBOOT patameter to reboot after flashing using -E

-added -BASE parameter to set ROM base address manually and allow

UniFlash to run without PCI bus

1.27       -added -MONO parameter for monochrome monitors

-enhanced -PCIOR functionality, you can now manually specify

a device by its BUS, DEVICE and FUNCTION numbers (decimal)

-fixed HiNT VXPro-II (Utron UC85C501/502) 128KB Flash ROMs

-added some VIA chipsets: ProSavage KM133(A), P4M266, P4X266(A)/333

-added real VIA VT8233 support

-fixed hang when EMM386.EXE is loaded (display error message now)

-success chime sound also in emergency mode

-fixed Atmel AT29Cxxx detection

-added ITE IT8700F/8705F and IT8710 LPC Super I/O detection with

SiS 540 and SiS 630 chipsets and all newer

-added some SiS chipsets : SiS 635, SiS 640, SiS 645, SiS 645DX,

SiS 650, SiS 730, SiS 735 and SiS 740 - I have no datasheets, but

SiS 735 is tested and works

-clear keyboard buffer before asking for file names and also on exit

-fixed memory de-allocation bug when HIMEM.SYS is loaded

1.26       -added Intel E7500 chipset with 82801CA (ICH3-S) I/O Controller Hub

-added some ALi chipsets: Aladdin III (M1521 and M1523(B)) and

Aladdin IV/IV+ (M1531(B))

-ALi FINALi 486 chipset (M1489) is not supported because it doesn't

map Flash ROM below 4GB

-added AMD 760MPX chipset with AMD-768 South Bridge

-added OPTi Vendetta 82C750 chipset (removed the old disabled code)

-added HiNT VXPro-II chipset (I think that it's Utron UC85C501/502,

because this one matches it and has programmable Mfg.&Device IDs)

-added -QUIT parameter - quits when processed, all comands after

will be ignored (useful use: "UNIFLASH -SAVE BACKUP.BIN -QUIT")

-added German language support (by Gerald Bottcher)

-PCI-to-ISA posted write buffer is disabled on ALi chipsets during

flashing to fix problems with some Flash ROMs (e.g. EON)

-added unprotect for SST49LF002A/003A/004A/008A FWHs

1.25       -fixed crash in Windows on some machines - display error message

(thanks to Alejando Pelaez)

-pressing ESC in -chiplist will quit

-CMOS size in log

-added -CMOSS (Save CMOS), -CMOSR (Restore CMOS) and -CMOSC (Clear

CMOS) options (they work also in Windows)

-added support for 256b CMOS

-fixed CMOS support to save/restore/clear also bytes 15 and 16

-added ServerWorks ServerSet III LE and HE chipsets with Reliance

NB6536 & NB6635 North Bridges (without any info from ServerWorks)

-added National Semiconductor PicoPower Vesuvius-LS chipset

-added National Semiconductor Geode GX1 CPU with North Bridge

(Cyrix MediaGX) and CS5530(A) South Bridge (Cyrix CX5530(A))

1.24       -added support for some ALi chipsets: Aladdin Pro IV and ALiMAGiK 1

-fixed 256KB and 512KB support for ALi chipsets

-added VIA Apollo VP3 with VT82C596 South Bridge

-implemented Redetect Flash ROM function

-AMI Flash Interface is no longer automatically enabled to prevent

hangs. You must use "-ami" parameter to activate it.

-added two more IDs for Macronix MX28F1000P - $4B and $D5

-simplified detection routine - removed support for multiple Flash

ROMs and removed all 16-bit wide ROMs, cleaned up the code

-pressing Q in -chiplist will quit (like MORE.COM)

-fixed Macronix MX29F002(N)T slow erase bug

-added Reliance/ServerWorks OSB4 South Bridge

1.23       -added Italian language support (by Enrico Antongiovanni)

-added timeout to GenPageProgB and GenPageProgW (to prevent hangs

with boot block protection enabled on Winbond Flash ROMs)

-added auto-logging feature (-LOG parameter)

-added support for Intel 830MP and 845 chipsets

-new keyboard handler doesn't use BIOS - this allows UniFlash to

work correctly under Award BootBlock BIOS (tested)

-added simple help on the bottom of screen in menu

-added new program/erase routines for Winbond Sectored Flash ROMs

(may fix Winbond W29F002U?)

-tried to fix but still not fixed SiS 501/5101/5501 support

-some VIA reverse engineering (reserved registers seems to be used)

but new VIA/AMD chipsets still not working (if you have "Flash

Write Protect" option in BIOS, your chipset will work when you

disable it)

1.22       -added error messages: running under Windows, PCI bus not found

-fixed Flash ROM write protect enable/disable - fixed possible

chipset register corruption

-added new delay routine based on Time Stamp Counter (TSC), it's

used on all CPUs that supports TSC (Pentium+), old CPUs use the

old routine. This fixed all problems with slow performance on

fast computers

-modified waiting times for Page Write Flash ROMs (most of Atmel,

SST and Winbond Flash ROMs) to prevent lock-ups

-fixed i430MX South Bridge detection (thanks to Josh Little but he

paid hard for that - his notebook doesn't work anymore because

Intel 28F004-T support doesn't work)

-fixed a _VERY_ NASTY BUG in Bulk Erase/Need Blanking chip support

this one was there probably from the beginning of UniFlash and

actually caused the same byte to be flashed to entire chip...

This fixed almost all Bulk Erase/Need Blanking chips.

-Nils Emil P. Larsen tested these Bulk Erase/Need Blanking chips,

they're working CAT28F010, MX28F1000P, Intel 28F010 and also H.T.

M21561 (which is actually MX28F1000P)

-added support for Intel 860 chipset (Xeon) and ALi Aladdin V

-added support for 1MB Flash ROMs with VIA VT82C596(A|B) and

VT82C686(A) South Bridges

-doesn't exit when no Flash ROM detected in PCI Option ROM mode

-chiplist now pauses after each screen (no need for external more

program) but not when stdout is redirected

-fixed memory corruption when using -chiplist (thanks to Marusceac

Claudiu Florin)

-beep after successful flashing

-heavy menu modifications, removed expert and debug modes - they're

not needed anymore, add Home,PgUp/End,PgDown keys

-removed BIOS info (I haven't seen it working anywhere)

-added some Intel chips: 28F008-T series and 28F008-B series

-added STMicroelectronics Flash ROMs: M28F256, M28F512, M28F101,

M28F102, M28F201, M29F512B, M29F010B, M29F040(B), M29F080A,

M29F016B, M29F002(B)(N)T, M29F002(B)B, M29F100(B)T, M29F100(B)B,

M29F200(B)T, M29F200(B)B, M29F400(B)T, M29F400(B)B, M29F800AT,

M29F800AB, M29F160BT, M29F160BB, M29W512B, M29W010B, M29W022BT,

M29W022BB, M29W040(B), M29W200BT, M29W200BB, M29W400(B)T,

M29W400(B)B, M29W800(A)T, M29W800(A)B, M29W160BB/DB, M29W004(B)T,

M29W004(B)B, M29W008(A)T, M29W008(A)B, M50FW040 (FWH),

M50FW080 (FWH) and M50LPW080 (LPC)

-added Texas Instruments Flash ROMs: TMS28F210, TMS29xF040 and

info about the others into docs

-added Fujitsu Flash ROMs: MBM29F002TC, MBM29F002BC, MBM29F200TC,

MBM29F200BC, MBM29F400TC, MBM29F400BC, MBM29F040C, MBM29F033C,

MBM29LV160TE, MBM29LV160BE, MBM29PL160TD and MBM29PL160BD

-modified Mosel support for SyncMOS chips F29C31004B and F29C31400T

-fixed a bit PCI Option ROM enabling (for AGP cards), but flashing

still does not work (looks like writes to ROM are ignored - like

write protection, the Flash ROM is not responding to commands)

-simplified the detection routine (note: new one is still not done)

1.21       -added support for SiS chipsets: 85C496+497, 85C501/5101/5501,

5511, 5571,5591/5592, 5596, 5597/5598/5581/5120, 530, 540, 600,

620 and 630 (all untested). It's unbelievable, but they are using

5 (!) different methods to enable Flash ROM write and what's more,

all their south bridges have the same device ID - $0008 (SiS 540

and 630 can have either $0008 or $0018).

-added support for SMSC VictoryBX-66 chipset

-added support for Winbond/Symphony Labs W83C553F South Bridge

-added support for ITE South Bridges IT8871F/2F and IT8888F

-fixed Flash ROM detection - Device IDs can have even parity

-changed sector structure array - can now hold more sectors (upto

5*65535), 128b to 8MB each (see addchip.faq)

-added support for Alliance Semiconductor Flash ROMs: AS29F040,

AS29F002T, AS29F002B, AS29LV400T, AS29LV400B, AS29F200T,

AS29F200B, AS29LV800T and AS29LV800B

-added some AMD Flash ROMs: Am29LV001BT, Am29LV001BB, Am29LV010B,

Am29F004BT, Am29F004BB, Am29LV040B, Am29LV081B, Am29LV008(B)T,

Am29LV008(B)B, Am29LV800BT, Am29LV800BB, Am29SL800CT, Am29SL800CB,

Am29F016(B), Am29F017B, Am29LV017B, Am29LV116BT, Am29LV116BB,

Am29F160DT, Am29F160DB, Am29LV160BT/DT, Am29LV160BB/DB,

Am29SL160CT, Am29SL160CB, Am29PL160CB, Am29F032B, Am29LV033C,

Am29LV320DT, Am29LV320DB, Am29LV065D

-added some Atmel Flash ROMs: AT29LV512, AT49x512, AT49x010,

AT49x001(N)T, AT49x001(N), AT49x002(N)T, AT49x002(N),

AT49x2048(A), AT49x4096(A), AT49x008AT, AT49x8192(A)T,

AT49x8192(A), AT49x8011T, AT49x8011, AT49x160/161(4)T,

AT49x160/161(4), AT49BV320T/321T, AT49BV320/321

-added some Catalyst Flash ROMs: CAT28F010, CAT28F020, CAT28F102,

CAT28F512

-added EON Flash ROMs: EN29F040, EN29F080, EN29F800T, EN29F800B,

EN29F002T, EN29F002B

-added Hyundai Flash ROMs: HY29F002T, HY29F002B, HY29F040,

HY29F040A, HY29F080, HY29F400T, HY29F400B, HY29F800T, HY29F800B,

HY29LV160T, HY29LV160B

-added IMT Flash ROMs: IM29F001T, IM29F001B, IM29F002T, IM29F002B,

IM29F004T, IM29F004B, IM29LV004T, IM29LV004B

-added some Intel chips: 28F256A, 28F512, 28F004Sx, 28F008Sx,

28F008SA, 28F016Sx, 28F016S5/SA, 28F320J3A, 28F320J5, 28F640J3A,

28F640J5, 28F128J3A

-added ISSI (Integrated Silicon Solution) Flash ROMs: IS28F010 and

IS28F020

-removed Macronix MX28F1000PC and MX28F1000PPC (not existing)

-added some Macronix chips: MX28F2000P, MX28F2000T, MX29F100T,

MX29F100B, MX28F002T, MX28F002B, MX29F001T, MX29F001B,

MX29F002(N)T, MX29F002(N)B, MX29F022T, MX29F022B, MX29F200T,

MX29F200B, MX29F004T, MX29F004B, MX29F400T, MX29F400B, MX29LV004T,

MX29LV004B, MX29LV400T, MX29LV400B, MX29F040, MX29LV040, MX29F080,

MX29LV081, MX29F800T, MX29F800B, MX29LV008T, MX29LV800T,

MX29LV800B, MX29F016, MX29L1611, MX29L1611G, MX29F1610A,

MX29LV160T, MX29LV160B, MX29F1615, MX29L3211

-added Mosel Vitelic Flash ROMs:

V29C51000T, V29C51000B, V29C51001T, V29C51001B, V29C51002T,

V29C51002B, V29C51004T, V29C51004B, V29C51400T, V29C51400B,

V29C31004T, V29C31004B, V29LC51000, V29LC51001, V29LC51002

-added PMC Flash ROMs: Pm39LV512R, Pm39LV010R, Pm29F002T,

Pm29F002B, Pm29F004T/5V or Pm29LV104T, Pm29F004B or Pm29LV104B

-added some SST Flash ROMs: 29SF512, 29VF512, 29SF010, 29VF010,

29SF020, 29VF020, 29SF040, 29VF040, 39SF512, 39xF512, 39S010(A),

39xF010, 39SF020(A), 39xF020, 39SF040, 39xF040, 39xF080, 39xF016,

39xF200A, 39xF400A, 39xF800A, 39xF160, 49LF002(A), 49LF003A,

49LF004, 49LF004A, 49LF008, 49LF008A, 49LF020, 49LF030, 49LF040,

49LF080

-added some Winbond Flash ROMs: W29C040, W49F002U, W49V002(A),

W49V002F(A), W29F102 or W49F102, W49L102, W49F201, W49L201

-some code size optimizations

1.20       -added support for these AMIC chips: A29001(1)T, A29001(1)B,

A29010, A29002(1)T, A29002(1)B, A29400T, A29400B and A29040

They're all sectored and use AMD commands.

(For Matt Choudoir's broken ABIT KT7A with AMIC A29002T)

A29002(1)T tested and works correctly, others are untested.

-added support for newer VIA chipsets (all untested)

Apollo MVP4, Apollo Pro, Apollo Pro Plus/133/133A,

Apollo ProMedia, Apollo Pro 133Z/PM133, Apollo KX133,

Apollo KT133(A), Apollo KT266, Apollo Pro 266.

North Bridges: VT8501, VT82C691/693(A)/694X, VT82C693(A), VT8601,

VT8605, VT8371, VT8363(A), VT8366, VT8633

South Bridges: VT82C686(A), VT8231, VT8233 (last two may or may

not work because I don't have datasheets).

Plus some north bridges that aren't used anywhere (but are in

VIA's INF file) - VT8653, VT8662, VT8615, VT8361, VT3133.

-added support for AMD 750, 760 and 760MP chipsets (751,761 and

762 north bridges, 756 and 766 south bridges). Registers are

almost same as VIA. Untested.

1.19       -added support for Intel 8xx chipsets (Hub Architecture):

i810, 810E, 810E2, 815, 815E(P), 815EM, 820, 820E, 840, 850

with these I/O controller hubs:

82801AA(ICH), 82801AB(ICH0), 82801BA(ICH2), 82801BAM(ICH2-M)

(they're all not tested)

-fixed ALi chipset detection

-M1543 now support 512KB ROMs, but only some revisions of this chip

can really support them (and according to ALi, there is no

software way to detect that)

1.18       -I (Rainbow Software) am continuing the development of this program

after it was abandoned for two years

-added support for i440BX/ZX and i440GX chipsets

-added support for ALi M1543 south bridge and ALi M1621 north

bridge (ALi Aladdin Pro II chipset)

-added some info about ASD chips into this DOC

-added support for Intel 82802AB(4Mb)/AC(8Mb)Firmware Hubs

-added support for Atmel AT49F002(N)T chip

1.17b      -fixed a bug in the -FORCE option, should work now

-still having trouble correctly detecting flash chips, we can use

all the input we can get on which chips are and aren't correctly

identified by UNIFLASH !

1.17       -pre-1.20 release because of some important fixes, still working

on some things we promised for 1.20, though

-fixed flashing problems with Macronix & AMD chips

-Integrated English & French version into one executable to avoid

having to keep 2 versions synchronized

-Cleaned up chipset detection

-known bugs: chip detection still not 100% accurate

(we're working on it, this will be fixed in v1.20)

1.16d      -Added AMI Flash Interface support

-Added the "Additional BIOS Info" option

-Added the Macronix MXIC 28F1000PPC/12V chip

(not officially released by Adam's request)

1.16c      -Made the detection routine more accurate

1.16b      -Removed some (debugging?) code that messed up detection

1.16       -Fixed a stupid bug in the chip detection part, which made it

impossible to identify Atmel chips, for example.

-Tried to add TI chips and after long hours of work and of

downloading datasheets figured out that TI chips are just

"remarked" AMD & Intel chips (the same Mfg & Id numbers), so TI

chips will be recognised as AMD or Intel ones.

-Made the source code clearer. Wrote a FAQ explaining how to add

your own chips.

-Added the Intel 28F010/12V and 28F020/12V chips

-Translation of the whole thing to french. The english 1.16 version

equals french 1.00FRENCH version

1.15       -Fixed problems with AMD flash chips

-Added VIA 82C570 chipset support

-Tested Intel 28F001BX-T/28F001BN-T support

TO DO before final release of V1.15: fix extended memory detection,

does not work ok on a friends' system ...

1.14       -Added Macronix support

-Added SST 28EE011 flash chip

-Added AMD Am28F010, Am28F010A, Am28F020, Am28F020A, Am28F256

Am28F256A, Am28F512 and Am28F512A flash chips

-Removed valid PCI ROM check so it is possible to flash a ROM on a

PCI card that does not (yet) contain a valid ROM

-Added some safety stuff:

-cache disabled during flashing (having the cache enabled seems to

be a problem with some Cyrix chips (or chip/chipset combo's ?))

-interrupts now fully disabled during actual communication with

the flash chip for increased reliability

-added multiple flash chips/single ROM image support (experimental)

1.13       -Added Catalyst support

-Added OPTi chipset support (for 82C750 'Vendetta')

-Added Intel 28F400/28F004 flash chips

-Completely rewrote PCI option ROM support (still experimental)

1.12       -Fixed problem ID-ing Intel chips (oops)

-Made first attempt at supporting PCI option ROMS (experimental)

1.11       -Added Atmel support

1.1        -Fixed memory allocation bug with sectored (Intel,AMD) flash ROM

chips

-Added VIA chipset support

1.0        -First public release

9.1            KNOWN BUGS

-------------------------

- Intel 28F004-T doesn't work

- PCI Option ROM Flashing (-pcior parameter) doesn't work on many cards

- 256KB BIOSes don't work on some i430VX/HX boards (AWDFLASH works!)

- Winbond W49F002U doesn't work - looks like write protected (sometimes???)

- Some VIA and AMD chipsets don't work - maybe because non-standard flash

write protection on some boards.

- Garbage on screen instead of flashing - on Asus P5A board with Aladdin V

and 256K Flash ROM, but it works OK on Micronics C200 (same chipset and

128K Flash ROM). On PC Partner VIB8x4DS board with VIA Apollo VPX, UniFlash

flashes the SyncMOS F29C51002T chip OK but then hangs with garbage on

screen. This problem appears to be associated with SyncMOS Flash ROMs only.

- Many Abit boards do not work - looks like undocumented write protection

9.2            What's missing

-----------------------------

- Boot Block on Winbond W29C020 can be flashed even if the Boot Block

protection is enabled on the chip - AMIFLASH v8.15.02 can do it but

UniFlash can't (I don't know how to do it)

- Support for ServerWorks, UMC, new VIA and new SiS chipsets because

datasheets are unavailable (these companies are simply ignoring e-mails)

I can't do much with only "unofficial" info.

- Support for some exotic Flash ROMs like Intel 82802AA 2mbit Firmware Hub or

Bright BM29FS040 (no datasheets available)

10.0           Compatibility tests

----------------------------------

Here is a list of tested & working motherboards/Flash ROM chip combinations.

Manufacturer  | Model + revision  | Chipset       | Flash ROM chip used

---------------------------------------------------------------------------

ASUSTeK       | T2P4 rev 2 & 3.10 | Intel 430HX   | SST 29EE010/5V

Expertboard   | 8661              | Intel 430VX   | Winbond W29EE011/5V

Soyo          | 5TA2              | Intel 430FX   | SST 29EE010/5V

Shuttle       | Hot-569           | Intel 430TX   | Atmel AT29C010A/5V

Shuttle       | Hot-565           | Intel 430TX   | Winbond W29EE011/5V

Shuttle       | Hot-603           | AMD 640       | Winbond W29EE011/5V

ASUSTeK       | P2B rev 1.10      | Intel 440BX   | Winbond W29EE011/5V

PC Chips      | M726 v3.4A        | Aladdin Pro 2 | ASD AE29F1008/5V

Abit          | BX6 rev. 2        | Intel 440BX   | AMIC A29002T/5V

MicroStar MSI | MS-5120 ver 1     | Intel 430FX   | Winbond W29EE011/5V

Abit          | BE6-R2            | Intel 440BX   | Winbond W29C020/5V

DFI           | P2-XLX            | Intel 440LX   | Winbond W29EE011/5V

FIC           | PT-2006           | Intel 430VX   | Intel 28F001/12V

PC Chips      | M726 v3.4A        | Aladdin Pro 2 | Winbond W29C020/5V

Abit          | BX6-R2            | Intel 440BX   | Winbond W29EE011/5V

Abit          | BX6-R2            | Intel 440BX   | Winbond W29C020(C)/5V

Abit          | BX6-R2            | Intel 440BX   | SST 29EE010/5V

Abit          | BX6-R2            | Intel 440BX   | CAT28F010/12V

Abit          | BX6-R2            | Intel 440BX   | Macronix MX28F1000P/12V

Abit          | BX6-R2            | Intel 440BX   | Intel 28F010/12V

AOpen         | AP5C/P            | Intel 430FX   | SST29EE010/5V

Gigabyte      | GA586SG           | SiS 5591+5595 | Atmel AT29C010A/5V

Chaintech     | 5IFM0             | Intel 430HX   | Macronix MX29F001B/5V

ASUSTeK       | P2B rev 1.10      | Intel 440BX   | Atmel AT49F002T/5V

Micronics     | C200              | Aladdin V     | SST 29EE010/5V

MicroStar MSI | MS-5120 ver 1     | Intel 430FX   | SST 29EE010/5V

MicroStar MSI | MS-5120 ver 1     | Intel 430FX   | Macronix MX28F1000P/12V

MicroStar MSI | MS-5120 ver 1     | Intel 430FX   | H.T. M21032T/12V

PC Chips      | M726 v3.4A        | Aladdin Pro 2 | Macronix MX29F002(N)T/5V

PC Chips      | M726 v3.4A        | Aladdin Pro 2 | EON EN29F002NT/5V

Mercury/Kobian| KOB 694X FSX r1.0 | ApolloPro 133A| SST 39VF020/3V (PLCC32)

QDI           | LegendX           | Intel 440LX   | Winbond W29C020(C)/5V

PC Chips      | M741LMRT          | SiS 620+5595  | SST 29EE010/5V

PC Partner    | VIB8x4DS          | Apollo VPX    | SyncMOS F29C51002T/5V

Soyo          | 5EMA+ v1.1        | Apollo MVP3   | Macronix MX29F001T/5V

LuckyStar     | K7VAT+            | Apollo KT133  | Winbond W29EE011/5V

LuckyStar     | K7VAT+            | Apollo KT133  | Winbond W29C020(C)/5V

LuckyStar     | K7VAT+            | Apollo KT133  | SST 39SF020(A)/5V

Chaintech     | 7AIA5             | Apollo KT133A | Winbond W49F002U/5V

PC Chips      | M571              | SiS 5598      | Winbond W29EE011/5V

PC Partner    | P5VB3+ Rev. B     | Apollo MVP3   | AMD Am29F010A/5V

Mercury/Kobian| KOB 630E FSFx r1.1| SiS630+IT8705F| EON EN29F002T/5V

ECS           | K7S5A             | SiS735+IT8705F| Winbond W49F002U/5V

PowerTech     | MB532 v1.0        | Intel 430FX   | SST 29EE010/5V

Matsonic      | MS6260S rev.1.6A  | Aladdin V     | ASD AE29F1008/5V

PC Partner    | TXB820DS          | Intel 430TX   | Atmel AT29C010A/5V

PC Chips      | M570 v1.3         | SiS 5591+5595 | Atmel AT29C010A/5V

Gemlight/DTK  | GMB-P57IPS-V1     | Intel 430TX   | Winbond W29EE011/5V

PC Chips      | M918 V1.2A        | FinALi 486    | H.T. M21032T/12V

PC Chips      | M918 V1.2A        | FinALi 486    | Macronix MX28F1000P/12V

PC Chips      | M918 V1.2A        | FinALi 486    | Winbond W29EE011/5V

PC Chips      | M918 V1.2A        | FinALi 486    | Atmel AT29C010A/5V

GVC/BCM       | FR500             | SiS 5598      | AMD Am29F002(N)(B)T/5V

Asus          | A7V266-E          | Apollo KT266A | SST 49LF020/3V

PC Chips      | M537DMA           | Apollo VPX    | H.T. M20271H/12V

QDI           | BrillianX 1       | Intel 440BX   | SST 29EE020/5V

11.0            Beta testing stuff

----------------------------------

We would like to extend our thanx to the following people who've been

very helpful with beta testing this program.

NOTE: YOU can be on this list too! All you have to do is send

information on a system where UNIFLASH doesn't work and help with

getting it to work on that system.

Please help us in making UNIFLASH the universal and reliable tool we had

in mind when we started this project. Thanx in advance!

The heroes gallery (random order)

---------------------------------

Enrico Antongiovanni

Wim Bervoets

Michael L. Croswell

Polyvios Simopoulos

Matthias David Siebler

Matt Choudoir

Nils Emil P. Larsen (really big thanks)

Josh Little

Tom Wiener

Dennis Schulz (BIG thanks - provided 5 chips and wanted only

one of them back!)

Miha Ovcak

Alejando Pelaez

RusH

David Russell

Highlander

12.0            Biz support

---------------------------

We would like to thank the following people at the various flash ROM

and chipset manufacturers for their support (in random order):

Scotts Chen at Macronix International Co., Ltd.

Darlene Browne at Acer Laboratories, Inc., (USA), Administrator

Niel Smith, Application Engineering Manager, ALI USA

Henry Ho at Macronix International Co., Ltd.

Michael Solinas at Silicon Storage Technology, Inc. (SST), USA

Phillip Chang at Megawin Technology Co., Ltd.

Thanks goes to Intel - all datasheets are freely available on their web page

upon release of new hardware. Guess why are Intel chipset the most compatible.

The companies in the list below should take this as an example of developer

support.

I would like NOT to THANK TO following COMPANIES for providing NO SUPPORT

for independent developers and NOT REPLYING TO E-MAILS:

Silicon Integrated Systems (SiS)

United Microelectronics Corporation (UMC)

ServerWorks

I finally got some real e-mail from VIA, but only bad news:

"At this time, datasheets are only available to manufacturers under non

disclosure agreement who have a genuine application they are developing. This

policy is currently under review but at this time I cannot provide the

datasheets to you."

============================== END OF DOCUMENT ===============================